Dynamic RUL Prediction of LFP Batteries
Complete Manuscript Tables (Table 1 to Table 8)

Table 1 Physics-Informed Feature Set and Computation Formulas

| Symbol | Mathematical Definition | Physical Meaning & Degradation Link | Computation Formula |
| --- | --- | --- | --- |
| k | Cycle Number | Tracks baseline temporal aging progression and cumulative operational charge-discharge duration of the cell. | Current operational cycle index k at checkpoint |
| SOH | State of Health | Normalized remaining capacity ratio; primary macro-level health indicator across cell form factors. | Q_dis(k) / Q_nominal |
| IR | Internal Resistance | Measures ohmic + SEI conduction resistance; primary indicator of electrolyte decomposition and interfacial thickening. | Mean(dV / dI) during pulse at 10% SOC |
| T_mean | Mean Discharge Temp | Monitors thermal stress during operational discharge; drives Arrhenius aging acceleration and SEI growth. | Mean surface/core temperature over W = 10 |
| ΔSOH | Capacity Fade Rate | Normalized rate of capacity loss over lookback window; measures local degradation velocity (ΔSOH). | [Q_dis(k-10) - Q_dis(k)] / Q_nominal |
| ΔQ_log_var | IC Curve Log-Variance | Captures thermodynamic peak broadening in incremental capacity curve; acts as an early warning sentinel. | log[Var(dQ / dV)] over window W=10 |
| ΔQ_min | IC Curve Min Shift | Maximum localized downward shift (valley) in the differential capacity profile over the rolling lookback window. | min[Δ(dQ / dV)] over window W=10 |
| ΔQ_mean | IC Curve Mean Shift | Average baseline shift of incremental capacity curve; tracks global thermodynamic drift and stoichiometric imbalance. | mean[Δ(dQ / dV)] over window W=10 |

Table 2 Rolling Lookback Window (W) Sensitivity Analysis

| Window Size (W) | MAE (Cycles) | Accuracy (R2) | Std Dev (σ) | 90% Safety Bracket | Remarks |
| --- | --- | --- | --- | --- | --- |
| W = 5 Cycles | 81.68 Cycles | 81.27% | 98.90 Cycles | ±124.00 Cycles | Too sensitive to weather changes. |
| W = 10 Cycles (Chosen) | 81.68 Cycles | 78.77% | 99.35 Cycles | ±122.00 Cycles | Best historical memory depth. |
| W = 15 Cycles | 79.55 Cycles | 79.44% | 97.40 Cycles | ±122.00 Cycles | Smooth calculation across normal driving. |
| W = 20 Cycles | 76.87 Cycles | 81.17% | 94.20 Cycles | ±119.80 Cycles | Filters out voltage sensor noise. |
| W = 50 Cycles | 73.33 Cycles | 82.67% | 88.10 Cycles | ±116.40 Cycles | Hides sudden drops in real driving. |

Table 3 LightGBM Hyperparameter Tuning Experiments

| Config | Trees | Leaves | LR | MAE Error | Accuracy (R2) | Std Dev (σ) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Config 1 (Underfit) | 100 | 15 | 0.10 | 90.99 Cycles | 76.65% | 111.20 Cycles | Too simple; misses sudden drops. |
| Config 2 (Overfit) | 600 | 127 | 0.01 | 75.92 Cycles | 81.23% | 92.40 Cycles | Too heavy for cheap car chips. |
| Config 3 (Unregularized) | 200 | 63 | 0.20 | 78.76 Cycles | 79.90% | 96.80 Cycles | Jumpy countdown on rough roads. |
| Config 4 (Chosen) | 300 | 31 | 0.05 | 81.35 Cycles | 78.52% | 99.35 Cycles | Best industrial car chip choice. |
| Config 5 (Heavy Model) | 500 | 31 | 0.05 | 80.17 Cycles | 78.61% | 98.10 Cycles | Slightly better, but slower chip. |

Table 4 Checkpoint Polling Interval Evaluation Table

| Interval | Samples | MAE Error | Accuracy (R2) | Std Dev (σ) | 90% Bracket | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 5 Cycles (Chosen) | 4,234 | 81.35 Cycles | 78.52% | 99.35 Cycles | ±122.00 Cycles | Best balance of speed and safety. |
| 10 Cycles | 2,124 | 80.68 Cycles | 79.02% | 98.12 Cycles | ±118.50 Cycles | Good, but delays dashboard alerts. |
| 15 Cycles | 1,421 | 82.11 Cycles | 78.44% | 101.40 Cycles | ±121.70 Cycles | Slightly less precise during drops. |
| 20 Cycles | 1,070 | 79.34 Cycles | 79.85% | 97.05 Cycles | ±116.00 Cycles | Leaves 2-3 week blind spots. |
| 50 Cycles (Worst) | 435 | 88.54 Cycles | 77.32% | 108.90 Cycles | ±130.00 Cycles | Poor; lags badly and misses drops. |

Table 5 Unseen Test Cohort Validation Performance

| Dataset Split | Cell Count | Checkpoints | MAE Error | R2 Accuracy |
| --- | --- | --- | --- | --- |
| Training Set | 100 Cells | 18,240 | 48.70 cycles | 95.74% |
| Unseen Test Set | 24 Cells | 4,234 | 81.35 cycles | 78.52% |

Table 6 Empirical Benchmarking of Predictive Models on Unseen Test Cohort

| Model | MAE (Cycles) | R2 (%) | Flash Size | MCU Loop | Automotive ECU Feasibility |
| --- | --- | --- | --- | --- | --- |
| Linear Regression | 152.90 | 56.12% | 1.2 KB | 0.22 ms | Deployable but high error (>150c). |
| Random Forest | 80.18 | 79.15% | 28.1 MB | 1.85 ms | FAILED: Exceeds 1 MB Flash limit. |
| XGBoost Regressor | 82.53 | 80.25% | 1.36 MB | 0.68 ms | FAILED: Exceeds 1 MB Flash limit. |
| LightGBM (Ours) | 79.91 | 81.62% | 809 KB | 0.41 ms | OPTIMAL: Fits Flash, <1.5 ms loop. |

Table 7 Prognostic Maintenance Alert Classification Performance Metrics

| Performance Metric | Mathematical Definition | Numerical Calculation | Result |
| --- | --- | --- | --- |
| Accuracy | (TP + TN) / (TP + TN + FP + FN) | (498 + 3600) / (498 + 3600 + 72 + 64) | 96.79% |
| Precision | TP / (TP + FP) | 498 / (498 + 72) | 87.37% |
| Recall | TP / (TP + FN) | 498 / (498 + 64) | 88.61% |
| F1 Score | 2 · (Precision · Recall) / (Precision + Recall) | 2 · (0.8737 · 0.8861) / (0.8737 + 0.8861) | 87.98% |

Table 8 Hardware Microcontroller Benchmarking & Timing Breakdown

| Execution Step | Operations Performed | Measured Latency | Hardware Microchip Feasibility |
| --- | --- | --- | --- |
| 1. Sensor Data Acquisition | Reading V, I, T buffers | 0.12 ms | Direct CAN bus register read |
| 2. Feature Extraction | Rolling dQ log var (W=10) | 0.42 ms | Simple integer rolling variance |
| 3. Tree Evaluation | Traversing 300 trees | 0.41 ms | Integer IF/ELSE threshold logic |
| TOTAL INFERENCE | Full End-to-End Prediction | 0.95 ms | Leaves >99.9% CPU free (<1.5 ms budget) |