V. METHODOLOGY

A. LightGBM Model Architecture and Tabular Rationale

Reliable prognostics on automotive embedded devices demand machine learning models that optimize both predictive accuracy and computational efficiency. Although Long Short-Term Memory architectures have proven their effectiveness in predicting battery lifetime, they rely on complicated matrix multiplication when processing input streams of data. Recurrent networks with deep layers create unnecessary memory overhead and have high inference time on automotive control units. Contrarily, gradient boosting of decision trees, particularly LightGBM model, show better results when applied to structured tabular data. LightGBM works on histogram-based binning of continuous features and uses leaf-wise tree building with depth limit, which results in much higher speed of inference compared to neural networks.

In order to maximize the efficiency of the model on vehicle hardware under noise, hyperparameter tuning experiments were carried out in five different configurations. As it is presented in Table II, the first configuration with 100 trees, 15 leaves, and learning rate 0.10 was an example of severe underfitting, and its MAE of 90.99 cycles was unable to capture abrupt capacity declines. On the other hand, Config 2 with 600 trees and 127 leaves overfitted training noise, producing complex structures that were not appropriate for automotive hardware. Finally, Config 4 with 300 estimators, 31 maximum leaves, and a relatively small learning rate of 0.05 was chosen as the best industrial configuration with 78.52% of accuracy (MAE 81.35).

Table II: LightGBM Hyperparameter Tuning Experiments

| Config | Trees | Leaves | LR | MAE Error | Accuracy (R^2) | Std Dev (σ) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Config 1 (Underfit) | 100 | 15 | 0.10 | 90.99 Cycles | 76.65% | 111.20 Cycles | Too simple; misses sudden drops. |
| Config 2 (Overfit) | 600 | 127 | 0.01 | 75.92 Cycles | 81.23% | 92.40 Cycles | Too heavy for cheap car chips. |
| Config 3 (Unregularized) | 200 | 63 | 0.20 | 78.76 Cycles | 79.90% | 96.80 Cycles | Jumpy countdown on rough roads. |
| Config 4 (Chosen Optimal) | 300 | 31 | 0.05 | 81.35 Cycles | 78.52% | 99.35 Cycles | Best industrial car chip choice. |
| Config 5 (Heavy Model) | 500 | 31 | 0.05 | 80.17 Cycles | 78.61% | 98.10 Cycles | Slightly better, but slower chip. |

B. Grouped Leave-Cells-Out Validation

The standard method of k-fold cross-validation causes significant leakage in the time-series battery data. If each cycle of the same physical battery is randomly allocated to either the training or the test set, the model will obtain high accuracy due to interpolation between the neighboring cycles. To ensure that no leakage occurs, we use GroupShuffleSplit validation on the level of physical cells. Specifically, 124 commercial LFP cells are split into 100 cells used for training and 24 cells for testing. As shown in the training flow architecture (Fig. 2), the testing vault is completely out of reach during model training and tuning.

C. Conformal Prediction for Uncertainty Quantification

Point estimates for the remaining useful life are not informative enough for safety-sensitive vehicle controller algorithms without mathematical quantifications of uncertainty bounds. For reliable systems, the method uses split conformal prediction to generate valid prediction intervals. Split conformal prediction algorithm tunes nonconformity scores using out-of-fold calibration residuals separate from the 24 test cells. In particular, the absolute prediction errors in each validation cell are used to calculate an empirical 90th-percentile error quantile of 122 cycles. The calibrated bound defines a safety interval of ±122 cycles around the LightGBM point estimate. Different from Bayesian neural networks with Gaussian assumptions about error distribution, conformal prediction guarantees valid coverage even for trajectory skewness.

D. Checkpoint Polling Interval Selection

The question about the frequency of the execution of diagnostic polling by the onboard Battery Management System requires a compromise between the microcontroller computation and trajectory reactivity. Diagnostics in every cycle is an unnecessary computation waste on static plateaus, while insufficient polling leads to gaps in diagnostics. The following analysis of polling intervals was conducted for 22,474 evaluation checkpoints with 5, 10, 15, 20, and 50 cycles.

Table III: Checkpoint Polling Interval Evaluation Table

| Interval | Samples | MAE Error | Accuracy (R^2) | Std Dev (σ) | 90% Bracket | Simple Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 5 Cycles (Chosen) | 4,234 | 81.35 Cycles | 78.52% | 99.35 Cycles | ±122.00 Cycles | Best balance of speed and safety. |
| 10 Cycles | 2,124 | 80.68 Cycles | 79.02% | 98.12 Cycles | ±118.50 Cycles | Good, but delays dashboard alerts. |
| 15 Cycles | 1,421 | 82.11 Cycles | 78.44% | 101.40 Cycles | ±121.70 Cycles | Slightly less precise during drops. |
| 20 Cycles | 1,070 | 79.34 Cycles | 79.85% | 97.05 Cycles | ±116.00 Cycles | Leaves 2-3 week blind spots. |
| 50 Cycles (Worst) | 435 | 88.54 Cycles | 77.32% | 108.90 Cycles | ±130.00 Cycles | Poor; lags badly and misses drops. |

Table III and Figure 3 clearly demonstrate that the 5-cycle polling resolution represents the right compromise between responsiveness and performance. Though the sparse 20-cycle polling scheme shows a slight advantage in terms of mean error in case of stationary plateaus, it creates very dangerous “blind zones” for up to two or three weeks of road usage in the actual environment. At the end of the life period, LFP cells have non-linear capacity drops where the capacity falls below the safety level of 80% in 15 cycles. Thus, polling every 20 cycles makes it possible for the battery cell to fall to death without being checked by the algorithm.